Depth = Span x Depth Factor ÷ 100

Depth Factor takes into account the fact that the deck may not be solid as well as making sure there is enough depth to prevent too much deflection. It may not include the depth of the roadway.

Construction Costs

Construction Cost (£) = Span x Price/tonne x Density x Depth x Width ÷ 10

For bridges built on soft embankments, Total Construction cost will be 1.1 x Construction Cost (calculated from above). This takes into account the need for strong foundations.

Maintenance Costs

Repainting Cost (per time) (£) = Span x Price/m	Maintenance Cost (£) = Span x Price/m x design life

Total repainting costs = repainting cost per time x number of times required in design life.

Additional Notes

Arch 	– The concrete arch will need access under the bridge during construction, effectively closing the river to traffic. However, construction would be relatively fast.

- The steel arch is considered aesthetically attractive. Construction could be carried out without closing the river but would be relatively slow. Construction would be quicker if the river could be closed to traffic.

Cable-Stayed – The steel cable-stayed bridge is considered aesthetically attractive. The piers require pile driving operations which can be very noisy. This bridge also requires long approach spans. Construction would also be relatively slow.

Beam – Beam bridges are often considered less aesthetically attractive in larger structures. They are often better suited to lighter loads (i.e pedestrian/cyclist), especially at longer spans. Construction is relatively fast.

Box Girder – Box Girder Bridges are considered aesthetically similar to beam bridges but are more appropriate for larger loads and longer spans. Construction is relatively fast.

Truss – Truss bridges are considered to be aesthetically attractive especially in more complex structures. Simpler structures would be considered less attractive but would be quicker to construct.