Ɵ

Support structure

'arch'

Tie cable

L1

L2

Unless otherwise stated, give all answers to 1 decimal place and use g=9.81m/s2.

- Above is a simplified diagram of one half of a cable-stay bridge under construction. If L1 is the same as L2 what is the angle θ?

45˚

- Below is a free-body diagram of the arch from the above example. W represents the weight of the bridge and it acts at the centre of gravity, distance x from the pier. The distance x increases as the deck construction continues.

Ɵ

L

W

T

Rh

Rv

A

B

Using the angle found in the previous example, what are the angles A and B from this Diagram?

A = 45˚		B = 45˚

3. The free body diagram can be rotated to give a different view as shown below.

L/sinθ

W

T

Rh

Rv

x

A

B

θ

Find a general equation for the Tension T in terms of W, x and L.

(Hint: take moments about the support at A, you may need to resolve W and T into their vertical and horizontal components.)

Resolving T, vertical component = TsinA = Tsin45 = T/√2 = 0.71T

Resolving B, vertical component = WsinB = Wsin45 =  W/√2 = 0.71W

Taking moments about the pivot:

TsinA x L/sinθ = WsinB x X

Tsin45 x L/sin45 = Wsin45 x X	=> sin45s cancel out on left hand side

TxL = Wsin45 x X

T = WXsin45/L

4. If the Mass of the deck is 100g, what is the Weight in Newtons?

100g = 0.1kg		W = 0.1 x 9.81 = 0.981

5. Using this weight and your equation from question , complete the following table. Take L = 30cm.

either use both x and L in cm or convert both to m

T = WXsin45/L = 0.981Xsin45/30

| Distance from support x (cm) | Tension in cable T (Newtons) |
| --- | --- |
| 5 | 0.23 |
| 10 | 0.46 |
| 15 | 0.69 |
| 20 | 0.93 |
| 25 | 1.16 |

5. Using an A4 piece of cardboard, create the bridge deck model as shown below. Using a Newton meter as the tension cable, set up the experiment as shown in the diagram..

To make the bridge deck: Fold the A4 card in half and then into thirds to make a prism shape. Make holes in the positions below.

1cm

5cm

5cm

5cm

5cm

1cm

5cm

5cm

A						      			  B

Once you've made the holes, use sellotape to secure the prism shape (you'll probably need to re-make some of the holes through the sellotape).

20cm

Clamp stand

Newton Meter

Cardboard Bridge Deck

Thread the shaft of a clamp through hole A in the bridge deck, ensuring the hole is large enough that the deck can rotate freely. Loop one end of the Newton meter over another clamp 20cm higher up the clamp stand. Use the hook at the other end of the Newton meter to hold the bridge deck in the middle through hole B. Record the initial reading on the Newton meter. Use the the mass hooks to add weight at different distances along the deck and record your results in the table below.

Initial reading on Newton meter:

| Distance from support x | Newton Meter Reading (Newtons) | Corrected Reading T (Newtons) |
| --- | --- | --- |
| 5cm |  |  |
| 10cm |  |  |
| 15cm |  |  |
| 20cm |  |  |
| 25cm |  |  |

6. To find the corrected reading, subtract the initial reading you recorded at the start from the results you recorded during the experiment. For this experiment, we are interested in the increase of tensions in the cable due to the added weight, not the tension already present.

7. How well do your recorded results agree with your calculations?

8. What do you think may have caused any difference in results?

Possible reasons for differences (not exhaustive):

set-up of experiment not perfectly horizontal/vertical/correct angle.

distance along bridge deck not completely accurate

bridge deck construction not completely accurate

weight of bridge deck affecting results

Newton meter not sensitive enough – measuring errors

Some of force carried by friction at pivot

model used for calculation too simplified

Extension Questions

9. Redo your calculations and experiment using an 80g mass.

Weight in Newtons:

80g = 0.08kg	W = 0.08 x 9.81

T = WX/L = 0.08 x W / 0.3

| Distance from support x (cm) | Tension in cable T (Newtons) |
| --- | --- |
| 5 | 0.19 |
| 10 | 0.37 |
| 15 | 0.56 |
| 20 | 0.74 |
| 25 | 0.92 |

Initial reading on Newton Meter:

| Distance from support x | Newton Meter Reading (Newtons) | Corrected Reading T (Newtons) |
| --- | --- | --- |
| 5cm |  |  |
| 10cm |  |  |
| 15cm |  |  |
| 20cm |  |  |
| 25cm |  |  |

10. For this experiment we ignored the initial tension in the cable when we calculated the corrected readings. Why might it be important to know the initial value when designing real bridges?

Cables break when they reach a certain value of tension. If there is already some tension in the cable it will be able to carry less load.

e.g say the cable breaks at 60N, if there is no initial tension in the cable it can carry 60N of load, if it has initial tension of 30N it can only carry 30N of load before reaching breaking point.

11. Calculate the difference in your recorded results compared to your calculations as a percentage of your calculated results.